Intel x79 User Manual

Content

1 )Package -----------------------------------------------------------------------------2

- Brife Introduction---------------------------------------------------------------------------2

- Motherboard ----------------------------------------------------------------------3

- Install & design----------------------------------------------------------------------4

4.1 Jump set and slots-----------------------------------------------------------4

- Drive install-------------------------------------------------------------------------6

5.1 Drive program content---------------------------------------------------------6

5.2 Intel chipset install information----------------------------------------------6

5.3 Drive x11 Install----------------------------------------------------------------6

5.4 Sound drive install---------------------------------------------------------------6

5.5 VGA graphic outside drive program install--------------------------------7

- 6 Lan drive install------------------------------------------------------------------7

6)Bios setup---------------------------------------------------------------------------7

6.1 Main manual function---------------------------------------------------------8

- Package introduce: Please confirm the package if integrated, if any damaged, or shortage of accessory, please contact with your supplier.

A) Intel X79 motherboard *1pcs

B) SATA cable*2pcs

C)Manual*1pcs

D) Back panel 1*pcs

E) CD drive *1pcs

2 Introductions.

Chipset: Intel x79

CPU supported: LGA2011 , support Core i7(sandy Bridge-E) 1800mHZ,2133mHzprocessor.

Memory supported:DDR3 1066/1333/1600 DUAL DIMM, provide 4*240pin DDR3, the single DDR support 64GB max;

I/O features:

4*serial SATA2(The MAX speed 300MB/s)

1*serial port, compatiable16550UART model;

1*SATA3 slot (60mn/s)

8*USB Slots, compatible USB 2.0(2 inside)

8*USB Slots, compatible USB 3.0(2 inside)

1*PS/2 keyboard;

1*PS/2 mouse;

1*RJ45 1000M LAN

On-board HA Audio

Support 6 channel 3D surround sound output(such as ALC662 serial)

Support multi stereo sound

Provide sound input/Microphone/front horn/middle Sub-woofer out, horn output slot.

Sound source input: provide middle/sub-woofer sound output.

Sound source output: stereo horn or earphone sound source input

Microphone:

On-board slot : On-board RTL 8111el LAN

Expansion slot: 2* PCI Express x16,1*PCI Express x4,1*PCI, compatible PCI2.3

3 Motherboard

This picture just for reference, please based on actual object

- Install & setup

Notice: please check motherboard carefully, all marked 1 or white thin line that will be 1 leg place

4.1 Jump cable setup &slot

CLP-CMOS1:clear CMOS jump cable

| 1-2 | Clear Coms |
| --- | --- |

F_aUDIO1:Front panel output slot:

| PIN | function | PIN | Function |
| --- | --- | --- | --- |
| 1 | MIC+ microphone Slot | 2 | Ground connect with ground |
| 3 | Vbias microphone voltage | 4 | AuD_Vcc(AVCC) Sound card power |
| 5 | L_R_out right sound output | 6 | R_out back right output back |
| 7 | N.C non-connect | 8 | Key blank leg |
| 9 | L_L_out | 10 | L_out back left sound output back |

USB: Expansion slot:

| PIN | Function | PIN | Function |
| --- | --- | --- | --- |
| 1 | VCC: power Positive and negative pole | 2 | VCC: power positive and negative |
| 3 | D-: data negative signal | 4 | D-: data negative signature |
| 5 | D+: data positive signal | 6 | D+: data positive signature |
| 7 | GND: connect with ground | 8 | GND: connect with ground |
| 9 | Key | 10 | NC: blank |

Other Slots :

| PS/2(down) | PS/2 keyboard(purple) |
| --- | --- |
| PS/2(up) | PS/2 mouse(green) |
| R_USB2/3 | The first &second USB slot |
| E SATA R_USB1 | Outside SATA slot and the third slot |
| F_USB1 | The fourth USB convert slot |
| LAN | RJ45 slot |
| COMB | Serial COM connect |
| SPDIF | SPDIF connect slot |
| AUDI01/2 | 8 channel sound output/input/microphone slot |
| F_AUDI01 | Front sound slot |
| CD_IN1 | CD input slot |
| SATA2/3/4/5/6 | SATA slots |
| PWRATX1 | ATX power supply slot |
| ATX-12V | ATX 12V Power supply |
| CPU_FAN1 | Fan slot |
| Speaker1 | Speaker output |

Function slot F_panel1:

| Hard disk light(HDD_LED) | 1.3leg, leg1--power positive; leg 3--connect ground |
| --- | --- |
| Power light(PWR_LED) | 2.4leg:leg2--power positive; leg4 connect ground |

| ATX power button(PWR_SW) | 6.8leg:power on/off |
| --- | --- |
| Restart slot(RESET) | 5.7leg: restart button |

- Drive install :

5.1 drive install program content:

| Drive program content | Drive program description | The operated system use for |
| --- | --- | --- |
| X79 CHIPSET driver | Intel chipset install | INF-X79 |
| Tool/Directx11 | Direct x11 install | Dxsetup |
| X79 sound card Driver | Realtek serial sound drive program | Vista-win7-r267 wdw-r267 |
| X79 LAN driver | Intel chipset lan install | Win7 XP |

5.2 Intel chipset install :

Run<CD drive>:\x79 CHIPSET DRIVER\INF-X79 Nest step” go on.

Choose “yes” install go on.

Choose “next step” and go on

Choose “finish” restart computer, the system will find automatically, then finished the install.

5.3 Drive x11 drive program’s install

Run<Drive CD>:\TOOLS\DRIVER x11\ DXSETUP

Choose :”i accept” Then go on

Choose:”next step” , then go on

Then choose “finished”, and restart desktop motherboard, the system will find it automatically, signal install.

5.4 Sound drive program install.

Please find the sound chipset model in the motherboard before you install the drive program,

Then install as the chipset model .

5.4.1 Install sound drive program(take HD Audio serial install example)

Run<drive CD>:\x79 sound card driver\Vista-win7-r267,xp choose WDW-R267/ then choose  next step, go on. Finally choose finished, then the system will restart automatically

5.4.2 Offload sound drive program(take HD Audio serial example)

Start windows, then choose setup/control , then choose: add/delete program; Under add/delete program manual choose : realtek high definition Audio driver, press add/delete, the drive program will be delete automatically

5.5 Outside graphic card driver program install

Run outside graphic card<CD driver>: content setup.exe, press next, then go on, press yes, then go on, then press next step , then go on , then press finished, restart the system finish install.

5.6 Lan drive program install.

Run <CD drive>:X79 lan driver\win7,xp, choose XP.

Press “next step” then go on, choose” install “then go on .Choose” finished”, then restart the computer, the system will find it automatically, and the lithium battery on the motherboard will provide the battery.

- BIOS setup.

CMOS SETUP will install all the data on the motherboard CMOS SRAM. As the power off then  the lithium will provide the power for CMOS SRAM, the power is on, the BIOS will start POST(power on self test), press <DEL> button , then could come into AMI BIOS Cmos setup main screen.

| Main | Advanced | Pci pnp | Boot | Security | Chipset | Exit |
| --- | --- | --- | --- | --- | --- | --- |

6.1

- Main

System overview(system information) setup the time date, hard disk model and monitor model.

- Advanced

Advanced settings(higher bios function setup) setup BIOS provided special function, for example virus warning, open computer lead hard disk , and so on

- PCI PnP settings advanced PCI/PnP settings

Plug in and off use instance and PCI configure)

- Boot

Boot setting(this set manual including all the outside equipment setup, such as sound card, USB keyboard if open, use what kind PIO mode, and so on)

- Security

Security settings(BIOS code setup)

- Chipset

Advanced Chipset settings(higher chipset function setup)set up motherboard chipset related  configure, such as DRAM Timing and so on.

- Exit

Exit Options(exit setup) Including load less information/ overload problem and unsafe value/ give up change/no-save exit.

Load fail safe/Optimize defaults

Bios Security Features(setup administer/ user code)

Save Changes and Exit & discard changed and exit

- Outside SATA(e SATA) use.

1)use 1pcs SATA cable connect Esata2 slot and SATA1-5 any one, 2)connect E SATA hard disk to motherboard E SATA connect, and connect USB power or outside power 3)if connect under the motherboard running situation, please scan test hardware change in the setup management to find the E SATA hard disk, then could use. 4)if the signal of Esata hard disk is not stability, could come BIOS Esata ADJUST choose accordingly SATA CHANNEL , THEN MAKE THE SIGNAL STONG. 5)If can’t find the ESata as using , please take the ESATA cable and power cable out, then put again.